Exercise J3 – Defining Test Cases for Requirements (Traceability Continued)

- Task: We will take one of the applications from Exercise J1, choose at least 3 requirements from it, and extend the requirements traceability matrix by adding test cases for each requirement (with at least 2 test cases per requirement). We’ll also assign an artificial test status (e.g., Pass/Fail) for each test to simulate test execution results. For this exercise, I picked the scenario “New software release for an X-ray apparatus” as our application. This is a medical device software scenario, which allows us to demonstrate traceability in a regulated context. We will assume a few specific new requirements for this X-ray software release. Then we’ll create a RTM fragment for those requirements and develop test cases accordingly.

- Identify 3 requirements for the X-ray software (from the new release features):

- XR-R1: The system shall support a new imaging mode called “Low Dose Fluoroscopy” that reduces X-ray dose by 30% while maintaining image quality.

- XR-R2: The system’s user interface shall allow selecting the new imaging mode and display an indicator when Low Dose mode is active.

- XR-R3: The system shall prevent X-ray exposure if the protective door is open, displaying an error message, and log this event (this might be an existing safety requirement we want to re-verify in this release).

We will trace these requirements to test cases. Each requirement will have at least two test cases. We choose two to cover different aspects or scenarios.

- Define test cases for each requirement.

- For XR-R1 (new imaging mode reduces dose), one test might be to measure the dose output in the new mode vs. standard mode (to verify the 30% reduction), another might be to check image quality metrics or that the system actually produces an image in Low Dose mode.

- For XR-R2 (UI selection and indicator), we can test the UI control and check that the indicator light/label appears when mode is active, and maybe test that switching modes updates the indicator appropriately.

- For XR-R3 (safety interlock on door), test cases should include trying to initiate an X-ray with the door open (expecting it to be blocked and error shown), and a normal case with door closed (X-ray proceeds). Also possibly checking the log file for the recorded event.

We will assign IDs to test cases (e.g., TC1, TC2, etc. for X-ray tests), and record a Status (Pass or Fail). Since it says “artificial test status,” we can fabricate a scenario.

- Populate the extended Traceability Matrix with Requirements, Test Cases, and Status:

| Req ID | Requirement (summary) | Test Case ID | Test Description | Test Status |
| --- | --- | --- | --- | --- |
| XR-R1 | Low Dose Fluoroscopy mode reduces dose ~30%. | TC1 | Measure X-ray dose output in Low Dose vs Normal mode. Expect ~30% reduction. | Pass (dose reduced 32%) |
|  |  | TC2 | Capture image in Low Dose mode and in Normal mode, compare image quality (noise level within acceptable range). | Pass (quality acceptable) |
| XR-R2 | UI allows selecting Low Dose mode and shows indicator. | TC3 | On UI, select “Low Dose Fluoroscopy” mode. Verify mode is activated (check indicator on screen). | Pass |
|  |  | TC4 | Toggle from Low Dose mode back to normal mode. Verify indicator turns off and system returns to normal mode. | Pass |
| XR-R3 | Prevent exposure if door open, show error, log event. | TC5 | Attempt X-ray exposure with door sensor = OPEN. Expect no exposure, error message “Door open – X-ray inhibited.” | Pass |
|  |  | TC6 | Check system log after door-open attempt. Expect entry recorded about aborted exposure due to door. | Fail (no log entry found) |
|  |  | TC7 | (Control test) Attempt X-ray with door sensor = CLOSED. Expect exposure proceeds normally (image acquired). | Pass |